Laser cut Plate preparation

- Laser cut the base plate(/3D-model/3D print and Laser cut file) from a 6mm MDF.

  - 6mm MDF board

  - Cut: base plate laser cut.dxf

  - Cut: j1 moving plate laser cut.dxf

- Chamfer all the holes with a drill bit from the back side.

  - 12mm Drill bit

Wires preparation

- Solder the optic endstop wire to a 3 pin Dupont 2.54mm pitch connector. There’re total of 5 optic endstop wire.

  - 3 pin Dupont 2.54mm pitch connector housing x 5

  - Dupont 2.54mm pitch connector wire x 15

- Solder Joint 4(28HS33 gear ratio 1:20), Joint 5(35BYGH20), Joint 6 (28HS34) motor’s wire to a 4 pin Dupont 2.54mm pitch connector.

J6

J5

J4

Specified motors

  - Dupont 2.54mm pitch connector wire x 12(Red, Green, Blue, Black for readability)

  - 4 pin Dupont 2.54mm pitch connector housing x 3

Joint 1 assembly

3D printed parts

- j1 57 mount.stl

- j1 endstop block v2.stl

- Tap M5 threads into 8mm coupling

  - 8mm coupling

  - M5 Taps

- Install the Coupling on to the shaft of the NEMA 23 motor.

  - NEMA23 x 56mm

  - Tapped 8mm coupling

- Insert Hot Melt Insert Nut in () top and bottom side.

  - j1 57 mount

  - M5 Hot Melt Insert Nut x 10

- Pass the wire if NEMA 23 motor through the hole on the mount

- Bolt in the motor onto the mount

  - M5 x 18 flat head bolt x 4

  - M5 nut x 4

- Pass the endstop wire though the hole. Then bolt the optic endstop onto the mount.

  - Optic endstop x 1

  - 3 pin signal wire x 1

  - M3 x 22 round head x 2

  - M3 nut x 2

- Bolt endstop blocker onto j1 moving plate (chamfered side).

  - j1 endstop block v2

  - M3 x 12 round head bolt x 1

  - M3 nut x 1

- Bolt the j1 moving plate laser cut board onto the 8mm coupling (endstop blocker down).

  - M5 x 8 round head x 4

- Bolt the flat bearing on to the contraption

  - 120mm flat bearing

  - M5 x 18 flat head x 4